ELM Meter Reader - System Overview

How the ELM OCR service reads electricity meter photos for JalSoochak / FlowVision

| Version | 2.0 |
| --- | --- |
| Date | 21 October 2026 |
| Location | services/elm-ocr in the arghyam/flowvision repository |
| This document describes | The latest code, on branch elm-optimized |
| Deployed version | The code on the main branch. It judges photos the same way as the latest code, except that the latest code also rejects the history, billing and export screen labels (H, B1, EXPORT). The other differences are documentation, comments and tests; see the box below. |
| Audience | Managers, field and operations staff (Part 1) and engineers and operators (Part 2) |

About this version: what changed since version 1.0 (8 October 2026) In short: since version 1.0, hard-to-read photos go to a person instead of asking for a new photo; the service can now use either of two AI models, and Claude Sonnet 5.5 was chosen after a test; and a newer version, not yet live, also rejects a few more kinds of wrong screen. Technical detail of the changes: - 15 October 2026: photos that are hard to read (glare, an unclear digit, low confidence) now come back as REVIEW, with the model's reading for a person to check, instead of RETAKE. - 21 October 2026: the AI model that reads the photos is chosen with one setting (METER_PROVIDER): OpenAI GPT-6 Sol or Anthropic Claude Sonnet 5.5. A comparison on the same 100 photos the same day led to Claude Sonnet 5.5 being chosen (section 5). - Branch elm-optimized (not yet on main): screens labelled as a history register (H, H1 to H12), a billing register (B1 to B12) or an export register labelled EXPORT, EXP or EXPORTED are rejected as the wrong screen (export registers labelled with the code 2.8.x were already rejected). - Documentation cleanup: the README, docs/INTEGRATION.md and docs/DEPLOY_AWS.md were reorganised and code comments tidied; the evaluation scripts now require the photo set as arguments (--review and --images no longer default to a local trial folder). - Also on the branch: the example systemd unit uses the folder /opt/meter-reader/elm-ocr (on main: /opt/meter-reader/meter_reader_service), one unused function was removed, and the test suite has 261 tests (256 on main). - The service now lives in services/elm-ocr of the arghyam/flowvision repository and is served under /flowvision/v1/elm (and, unchanged, under the earlier /meter-reader/v1). This document describes the latest code (elm-optimized, a branch: a copy of the code with changes not yet merged into the main line). The deployed code is main, the main line; the only difference in how photos are judged is the H / B / EXPORT label rule above. All results measured on 21 October 2026 were produced with the main code; earlier results quoted from the README (section 4 and the end of section 6) were produced with older rules.

How to read this document

| Part | For | What it covers |
| --- | --- | --- |
| Part 1 - For everyone (sections 1 to 8) | Managers, field and operations staff, anyone new to the service | What the service does, how a reading works, the four possible answers, why it is careful, the AI model, today's results, privacy and limitations. Plain language, no code. |
| Part 2 - Technical detail (sections 9 to 21) | Engineers, operators, integrators | Architecture, API, the reading pipeline, rules, meter types, protections, configuration, storage, deployment, testing, code layout and open decisions. Precise names and settings. |
| Appendices A to C | Everyone, for reference | Glossary, every reason code with its exact texts, and where to find more. |

Technical words in Part 1 are explained where they first appear, and the main ones are also in the glossary (Appendix A). Names written in this font are exact names from the code, the API or the settings; Part 1 readers can skip them.

Numbers in this document come from the code, its documentation and the evaluation run of 21 October 2026. No consumer names, consumer ids or photo ids are included.

Contents

Part 1 - For everyone

1. The problem and what the service does

2. How a reading works, step by step

3. The four possible answers

4. Why we are careful

5. The AI model: GPT-6 Sol or Claude Sonnet 5.5

6. How well it works: today's results

7. Privacy and data

8. Known limitations and what is next

Part 2 - Technical detail

9. Architecture and where it fits

10. API reference summary

11. The reading pipeline in detail

12. Screen, label and unit rules

13. Meter types and the decimal point

14. Previous readings, feedback and corrections

15. Protections and limits

16. Model providers and configuration

17. Storage, database and privacy

18. Deployment and operations

19. Testing and evaluation

20. Codebase guide

21. Decisions and open items for the team

Appendix A: Glossary

Appendix B: Every reason code

Appendix C: Where to find more

# Part 1 - For everyone

This part explains the service in plain language. It needs no technical background.

## 1. The problem and what the service does

JalSoochak is the central system that collects electricity meter readings, and FlowVision is the set of meter-reading services this one belongs to (another one reads water flow meters).

JalSoochak needs readings of electricity meters, and these readings arrive as photos taken on a phone in the field. A reading is used for billing, so it must be right. Photos taken in the field are often hard: in our hand-checked set of 268 field photos from Assam and Punjab, 68 (25%) could not be read even by a person, and 54 (20%) showed a different screen of the meter, not the total.

The ELM OCR service (OCR means reading text from a picture) is the part of FlowVision that reads these photos. It takes one photo of a digital electricity meter and returns the meter's cumulative reading: the running total of energy the meter has counted since it was installed, in kWh (kilowatt-hours, the usual unit of electrical energy) or kVAh (a second energy total that some meters are billed in). With the reading it returns an answer that says how far the reading can be trusted.

The service is built around one rule: a wrong reading is far worse than no reading. A reading is only marked safe to use (ACCEPT) when every check passes. Anything doubtful goes to a person (REVIEW) or asks for a new photo (RETAKE).

### Key facts

| Item | Value |
| --- | --- |
| What it reads | The cumulative energy total (kWh, or kVAh on meters billed in kVAh) from a photo of a digital (LCD) electricity meter. 8 meter models used in Assam and Punjab are known to it by name (section 13). |
| Who calls it | The JalSoochak central system: one photo per request, and the answer comes back in the same call. |
| AI model | Claude Sonnet 5.5 (Anthropic) was chosen on 21 October 2026 after a side-by-side test. The code default is still GPT-6 Sol (OpenAI), so a deployment uses Claude only once it sets METER_PROVIDER=claude. Sending real photos to Anthropic needs its own approval, whose status is not recorded in the repository (section 5). One setting chooses; only the chosen model reads a photo. |
| Time per photo | On the same 100 photos: Claude Sonnet 5.5 median 4.2 seconds, GPT-6 Sol median 23.5 seconds (the median is the middle value: half the photos were faster). On all 268 photos, Claude: median 4.2 seconds, slowest 14.7 seconds. A reading is cut off at about 100 seconds; callers should set their timeout to 120 seconds. |
| Cost per photo | On the same 100 photos: Claude Sonnet 5.5 averaged $0.0137 and GPT-6 Sol $0.0223. On all 268 photos, Claude: median $0.0136, highest $0.0208. Model spending is capped at $25 per day by default. |
| Possible answers | ACCEPT (passed every check), REVIEW (a person confirms or corrects the reading), RETAKE (take a new photo; two kinds: wrong photo, or no meter display). |
| Where it runs | Packaged as a container (a sealed package of the program that runs the same anywhere). Every change to the service on the main branch is tested automatically and the container is published to Amazon's container registry (ECR); putting it live is a separate, manual step. It is designed to run on Amazon's cloud (AWS: ECS Fargate, recommended, or EC2) or on Kubernetes, with readings kept in a PostgreSQL database and photos in Amazon S3 storage. The repository does not record where it is running today. Details in section 18. |
| Built with | Standard open-source software (the Python language). 261 automated tests check the rules with a stand-in for the AI, so they make no paid AI calls. |

## 2. How a reading works, step by step

Figure 1. One meter photo, from the phone to the answer

- Photo. A person photographs the meter's display in the JalSoochak app.

- Request. The JalSoochak central system sends the photo to the ELM service, with its own request id, the consumer's id and the time the photo was taken.

- Preparation. The service checks the request, turns the photo upright and shrinks it if it is large (to at most 2048 pixels on the longer side).

- Reading. The AI model reads the display one digit at a time. It also reports the label printed next to the number (for example CUM kWh or MD kW), the unit, the meter's make and model, which digits it is unsure of and how confident it is.

- Safety checks. Fixed rules, written in code and not left to the AI, check the answer: is this the total screen, is every digit clear, is the unit kWh or kVAh, does the number fit this meter model, and does it make sense after the consumer's previous reading?

- Answer. The service returns the reading with ACCEPT, REVIEW or RETAKE, and saves the request, the photo and the result.

- Back to the app. JalSoochak receives the answer in the same call and shows it in the app.

- Confirmation or new photo. For ACCEPT and REVIEW the app shows the reading and the person confirms it (or corrects it). For RETAKE the app asks for a new photo and shows a hint on how to take it.

Some requests are answered before the AI is called, at no cost: a request that is sent again with the same request id gets the stored answer, and the identical photo of the same consumer sent again within 7 days gets the earlier result.

## 3. The four possible answers

Every reading that completes ends in one of four answers: ACCEPT, REVIEW and two kinds of RETAKE. A request can also fail with an error, for example when the AI provider is down, the daily budget is used up or the service is busy; the error says whether to try again (section 10). The colours below are used throughout this document.

Figure 2. The four answers and what the app does with each

| Answer | Technical status | When | What happens next | Reading shown? |
| --- | --- | --- | --- | --- |
| ACCEPT | SUCCESS | Every check passed. | The app shows the reading and the person confirms it. It can be used for billing. | Yes |
| REVIEW | NEEDS_REVIEW | The photo was hard to read (glare, an unclear or possibly missing digit, low confidence), or the reading failed a consistency check (digit count, decimal point, previous reading), or there was nothing to cross-check it against and the AI was less than 95% sure. | The app shows the reading; the person checks it against the meter and confirms or corrects it. The confirmed or corrected reading should be sent back to the service as feedback; who does this is still to be decided (section 21, item 6). | Yes, whenever the model saw digits (a ? marks a digit it could not guess) |
| RETAKE (wrong photo) | UNCLEAR | Not the total screen (for example a tariff, bill-history or maximum-demand screen), or the kWh / kVAh unit is not visible, or it is the wrong unit. | The app asks for a new photo and shows the hint. | For information only, when digits were seen |
| RETAKE (no meter) | NOMETER | No meter display is visible, or it is blank or switched off. | The app asks for a new photo and shows the hint. | No (a short sentence instead) |

ACCEPT and REVIEW both show the reading, and in both cases the person using the app confirms it. The difference is how much the service has already checked: an ACCEPT passed every check; a REVIEW reading is the model's best reading and needs a person's eyes. The rule for billing is: bill only on ACCEPT, or on a reading a person has confirmed.

A RETAKE asks for a new photo, with a short hint for the person taking it. The hints are fixed texts:

| Why | Hint shown to the person taking the photo |
| --- | --- |
| No display visible | Take the photo straight at the meter's display, with the display switched on. |
| Wrong screen | Wait until the display shows the total kWh (or kVAh) reading, then take the photo. |
| No unit visible | Take the photo so the whole display is visible, including the kWh or kVAh label next to the reading. |
| Wrong unit for this consumer | Wait until the display shows the billed unit (kWh or kVAh), then take the photo. |

A photo without a meter is not an error: it is a normal answer (RETAKE, no meter).

## 4. Why we are careful

A missing reading costs a new photo. A wrong reading can mean a wrong bill, an unhappy consumer and lost trust. So every part of the service leans towards caution: when in doubt, it asks a person or asks for a new photo.

Electricity meters make this harder than it looks. Their displays cycle through several screens: the total, but also maximum demand (the highest power drawn in a period, in kW), voltage, the date, tariff totals (energy used in one time-of-day band only) and past bills. Several of these look just like the total. The digits are made of small bars (seven-segment digits), and glare can make a 3 look like an 8 or a 5 like a 6. The decimal point is a tiny dot that is easy to miss, or to imagine.

The protections, in plain words:

- The AI is told to be strict. Its instructions say, in capitals: "A WRONG READING IS FAR WORSE THAN NO READING." They add: "UNREADABLE is always an acceptable answer." It must only say it read the meter when it is certain of every character.

- Only the total screen counts. If the label next to the number marks a tariff total, a past bill, maximum demand, a prepaid balance, an export register coded 2.8.x, or (latest code only) a history register (H, H1 to H12), a billing register (B1 to B12) or an export register labelled EXPORT, EXP or EXPORTED, the answer is RETAKE, whatever the AI concluded.

- Every digit must be clear and the number of digits certain; the AI's confidence must be at least 90%. Otherwise a person reviews it.

- The unit must be visible and be kWh or kVAh. These meters show both totals, so a photo without the unit cannot tell which one it is.

- When the meter is recognised as one of the 8 known models (from the model code or make printed on the meter, if it is readable in the photo), or the consumer's meter settings name its type, the decimal point comes from that model's known format, not from the AI spotting a dot. Otherwise a reading is accepted only when the dot is clearly visible; otherwise a person reviews it. For 6 of the 8 known models the service also knows how many digits the display has, which catches a dropped digit (the L&T EM101+ and the AEW APL-sm346 have no fixed digit count).

- Each reading is compared with the consumer's previous trusted reading. A reading that is lower, or that rose by more than 300 units a day, goes to a person.

- A higher bar without a cross-check. When neither the digit count and decimal position (from the meter model or the consumer's settings) nor a previous reading is known, a reading is accepted only if every check passed and the AI is at least 95% confident; otherwise a person confirms it.

- People's corrections are remembered. An ACCEPT reading (unless marked wrong), a confirmed reading, or a corrected one sent with the right value becomes the consumer's trusted previous reading. A photo whose reading a person marked wrong or corrected is never read or accepted again.

- Money is protected too. A retry with the same request id, or the identical photo of the same consumer sent again within 7 days, is answered without paying for a new reading, and model spending stops for the day at a cap ($25 by default).

Why this matters is shown by an earlier test with GPT-6 Sol (2 October 2026, rules of that date): of 21 accepted readings, 20 were right; the one miss was a clear-looking photo read as 0016183.1 instead of 0016189.1, with the AI 98% confident. Confidence alone is not enough: a one-digit misread like this one (6 units below the true value) can still look like a normal increase, so the previous-reading check may not catch it either. The service's own checks accepted it. What protects against it is the person in the app comparing the reading with the meter before confirming it, which the app asks for on ACCEPT as well as on REVIEW.

## 5. The AI model: GPT-6 Sol or Claude Sonnet 5.5

The reading itself is done by a vision model: an AI that can look at a picture and answer questions about it. The service can use one of two, from two companies (called providers):

- GPT-6 Sol from OpenAI (the default in the code, and the model the service was first built and tested with);

- Claude Sonnet 5.5 from Anthropic (added on 21 October 2026).

One setting chooses (METER_PROVIDER: openai or claude). Both models get exactly the same instructions and the same prepared photo (for Claude, a very large photo is compressed a little further to fit Anthropic's 5 MB image limit), and their answers go through exactly the same safety checks, budget and storage. Each stored reading records which model produced it. Only the chosen model reads a photo: if it fails, the service does not try the other one.

### Today's comparison: the same 100 photos, the same code

On 21 October 2026 both models read the same 100 hand-checked field photos (54 readable, 26 unreadable, 20 showing another screen), with the same rules and one model call per photo:

| Measure | GPT-6 Sol | Claude Sonnet 5.5 |
| --- | --- | --- |
| RETAKE (new photo asked) | 50 (wrong screen 38, no display 12) | 41 (wrong screen 34, no display 7) |
| REVIEW (a person confirms) | 44 | 56 |
| ACCEPT (passed every check) | 6 | 3 |
| Right reading returned on the 54 readable photos (any answer) | 32 | 46 |
| Readable photos sent to RETAKE | 8 | 6 |
| Wrong readings accepted | 0 | 0 (see note) |
| Cost for the 100 photos | $2.23 ($0.0223 per photo) | $1.37 ($0.0137 per photo) |
| Median time per photo | 23.5 seconds | 4.2 seconds |

Note: one of Claude's accepted readings disagreed with the hand label, but the photo clearly shows the accepted value (0019712.7); the hand label ("unreadable") was wrong, so no wrong reading was accepted.

### Why Claude Sonnet 5.5 was chosen

- It returned the right reading far more often: 46 of 54 readable photos, against 32.

- It asked for fewer new photos: 41 retakes against 50, and 6 readable photos sent to retake against 8.

- It is cheaper: about $0.014 per photo against $0.022.

- It is much faster: a median of 4.2 seconds against 23.5 seconds.

- It accepted no wrong reading, like GPT-6 Sol; but with only 3 and 6 accepted readings, this test cannot compare the two models on accuracy (section 6).

The trade-off: Claude accepts on its own less often (3 against 6) and sends more photos to REVIEW (56 against 44). Because the app shows the reading for REVIEW too and asks the person to confirm it, this costs the person a careful check and a confirmation or correction, not a new photo.

Two things to complete before Claude reads real photos in production - The code's default is still openai. The deployment must set METER_PROVIDER=claude with an Anthropic key (ANTHROPIC_API_KEY), and remove METER_MODEL or set it to a Claude model. If the Anthropic key is missing, or METER_MODEL still names a GPT model, the service refuses to start; if METER_PROVIDER is not set, it keeps using GPT-6 Sol (section 16). - The README says photos may be sent to OpenAI (approval given), but sending them to Anthropic "needs its own approval before real photos are used". The status of that approval is not recorded in the repository (section 21).

## 6. How well it works: today's results

On 21 October 2026 Claude Sonnet 5.5 read all 268 photos of the private hand-checked set (field photos from Assam and Punjab, kept outside the repository). Each photo had been labelled by hand: 146 "readable" (a person can read the total), 68 "unreadable" and 54 "wrong screen". The code used was the deployed main code.

| Answer | Photos | Share |
| --- | --- | --- |
| ACCEPT | 9 | 3% |
| REVIEW | 142 | 53% |
| RETAKE: wrong screen (UNCLEAR) | 94 | 35% |
| RETAKE: no display (NOMETER) | 23 | 9% |
| Total | 268 | 100% |

The same results split by what the photo really showed:

| What the photo showed (hand label) | ACCEPT | REVIEW | RETAKE |
| --- | --- | --- | --- |
| Readable total (146) | 8 | 118 | 20 |
| Unreadable even for a person (68) | 1 (hand label wrong; see below) | 16 | 51 |
| Another screen (54) | 0 | 8 | 46 |

### What these numbers mean

- No wrong reading was accepted. Of the 9 accepted readings, 8 matched the hand label and the 9th is correct on the photo (the hand label was wrong).

- The right reading was returned on 119 of the 146 readable photos (82%), decimal point included, whatever the answer. So in most REVIEW cases the number shown is already right, and the person only has to check it against the meter and confirm it.

- Most retakes were right to ask. They were real other screens (maximum demand, voltage, the date screen of Punjab AEW meters, tariff and bill-history screens) or photos with no usable display (blank, taken from too far away, or glare so heavy no digit was legible).

- About 9 of the 117 retakes (in 268 photos) could have been avoided (section 8): 5 readable photos with a T1 label (Genus SAKSHAM-345 meters show 'T1 CUM kWh' on their total screen; 4 of the 5 also showed CUM) and 4 glare photos where the unit was read as kW or kVA. A proposed rule change would recover 4 of them.

- The other readable-labelled retakes. The table shows 20 photos hand-labelled readable that went to RETAKE. Some of them are the avoidable retakes above; the review of the results judged the others correct retakes: some photos the hand label counted as a reading actually show another screen, such as a past-bill value (for example, the same number appears on 6 photos of one consumer over 10 days). The 2 October run found the same kind of hand-label error (17 Assam tariff or bill screens counted as readings).

- Why readings went to REVIEW: UNREADABLE 101, LOW_CONFIDENCE (the AI not confident enough) 25, NO_CROSS_CHECK (nothing to cross-check the reading against) 16; 15 of those 16 had the right reading.

- Cost and speed: $3.68 for all 268 photos (median $0.0136, highest $0.0208 per photo); median 4.2 seconds, slowest 14.7 seconds; no errors.

### Honest caveats

- No reading history was used. Each photo was judged on its own, so the previous-reading checks did not apply. In production they would apply once a consumer has a trusted previous reading: they can turn a NO_CROSS_CHECK review into an accept and catch a dropped digit, but they do not help unreadable photos.

- Few accepts. 9 accepted readings are far too few to show that the accuracy target is met (at least 95% of accepted readings must be right); about 60 accepted readings with zero errors are needed, from a fresh hand-checked set.

- REVIEW readings can be wrong. 18 of the 118 readable photos sent to REVIEW showed a wrong number (a dropped or extra digit, or a shifted decimal point), almost all with the AI's confidence at 50% or lower. This is why the person must really compare the reading with the meter, and why feedback matters.

- 5 REVIEW results had no usable number (for example "??????").

- These runs used the deployed main code, which does not yet reject the H, B and EXPORT labels. The effect of that change on these numbers has not been measured.

Earlier results with GPT-6 Sol (from the README, older rules): on 100 trial photos the returned reading was right on 36 of 51 readable photos, and every ACCEPT was correct except one bill-history screen that the current instructions now decline. On 268 fresh photos (2 October 2026, $6.17), 20 of 21 accepted readings were right (95.2%) and the returned reading was right on 87 of 146 readable photos. These runs used different rules from today's, so they are not a like-for-like comparison.

## 7. Privacy and data

|  | What |
| --- | --- |
| Sent to the AI provider | Only the photo and the fixed instructions. The photo is turned upright and made smaller when needed; a photo that needs neither is sent exactly as received, including any data the phone stored inside the file (which can include the time and place the photo was taken). The consumer id and the other request details are never sent. |
| Stored by the service | Every photo that is read (or answered from an earlier result), unchanged, in Amazon's cloud file storage (S3) or a local folder, named by date and request id, with no consumer details in the name. Photos are kept unless the operators set photo storage to none. A database record of each reading request that is processed, and of refused requests that carry a request id. A few requests refused at the door, such as those with a wrong access key (API key) or an oversized photo, are not recorded (section 17). The record holds the details sent (consumer id, photo time, any extra fields), the reading, the answer and its reasons, the model, time and cost, and any feedback; the image data itself is not in it. |
| In the logs | Each request and response, which include consumer ids. Never the photo (at any level of log detail), and never passwords or access keys. |
| Never stored | The photo inside the logs; access keys in logs, answers or the database; and the hidden access part of photo links (the part after ?, which can hold access tokens). |

These are consumer photos, so the README sets four rules: never publish or share them; restrict access to the folder or bucket; set a retention period (for example an S3 rule that deletes photos after an agreed time); and remember that the logs contain consumer ids.

Approvals: sending photos to OpenAI has been approved. Sending them to Anthropic (Claude) needs its own approval before real photos are used; its status is an open item (section 21). The 21 October evaluation sent the 268 hand-checked field photos to Anthropic; whether that use was covered by an approval should be confirmed and recorded together with the approval for production.

## 8. Known limitations and what is next

### Known limitations

- Few automatic accepts. With Claude and no reading history, only 9 of 268 photos were accepted on their own; most readable photos go to REVIEW and need a person's confirmation. Previous readings and per-consumer meter settings help mainly the NO_CROSS_CHECK reviews (16 of 142 today, 15 of them with the right reading). The other reviews were UNREADABLE (101) or LOW_CONFIDENCE (25), which need a clearer photo or a person.

- Genus SAKSHAM-345 meters (Assam) show "T1 CUM kWh" on their normal total screen. The rule that rejects tariff labels treats T1 as a tariff total, so these photos are asked to be retaken (5 readable photos showed T1, 4 of them with CUM).

- Glare can make the model read the unit as kW or kVA instead of kWh (4 photos). These go to RETAKE; this is left as is, because loosening it would let real maximum-demand screens through.

- Glary date screens can look like a reading: 3 went to REVIEW showing a number like 17092.6, with low confidence (15% to 30%).

- REVIEW without a number: 5 REVIEW results had no usable number.

- Only 8 meter models are known. For other meters the decimal point must be clearly visible, and a dropped digit is caught only by a previous reading or by a digit count set in the consumer's meter settings (the same applies to the two known models whose digit count varies).

- One model, no fallback. If the chosen model declines or fails, the request fails with an error that says whether to retry.

- Without the consumer id the meter settings and the previous-reading check are skipped, so more readings go to review.

### What is next

- Set Claude Sonnet 5.5 in production and confirm the Anthropic data-sharing approval.

- Merge elm-optimized (H, B and EXPORT label rejection) into main and deploy it.

- Decide on the proposed T1 + CUM rule change (would recover 4 of the avoidable retakes) and on whether a REVIEW without a usable number should become a RETAKE.

- Name an owner for the REVIEW queue (the REVIEW results waiting for a person), so every REVIEW is confirmed or corrected through feedback.

- Fill in per-consumer meter settings where they are known.

- Run a larger fresh hand-checked evaluation to test the accuracy target (at least 95% of accepted readings right): about 60 accepted readings with zero errors are needed, which at the 21 October accept rate (9 in 268, without reading history) means roughly 1,800 photos, unless reading history or meter settings raise the accept rate.

# Part 2 - Technical detail

This part is for engineers, operators and integrators. Names in monospace are exact identifiers from the code, the API or the configuration. File paths are relative to services/elm-ocr unless stated.

## 9. Architecture and where it fits

Figure 3. Components of the ELM OCR service and the systems it talks to

ELM OCR is one service in the FlowVision repository: services/bfm-ocr reads bulk flow (water) meters and services/elm-ocr reads electricity meters. Each service has its own code, dependencies, Docker image and CI workflow, and services never import from each other. ELM has its own URL prefix (/flowvision/v1/elm), its own ECR image repository (meter-reader) and its own database tables.

The call is synchronous: the JalSoochak central system sends one HTTP request per photo and gets the reading in the response. The only time the service contacts JalSoochak is to download a photo from the link it was given.

| Component | Technology | Role |
| --- | --- | --- |
| JalSoochak central system | External caller (HTTPS) | Sends each photo with its details and the X-API-Key header; uses the answer; sends feedback. |
| Load balancer / ingress | AWS Application Load Balancer, or a Kubernetes ingress | HTTPS in front of the service; idle timeout 120 s. API Gateway must not be used (its 29-30 s limit is shorter than a slow reading). |
| API layer | FastAPI app (app_factory.py) under gunicorn with uvicorn workers | API key and body-size checks before the body is read, request validation, endpoints, the FlowVision response envelope, /health and /ready. |
| Reading service | service/api/reading_service.py | Request id (idempotency), read slots, photo download, duplicate-photo check, daily budget, calling the reader, saving the photo and the record. |
| Vision reader | service/vision/ (reader.py, prompt.py, checks.py, meter_types.py, images.py) | Image preparation, the model call, the safety checks and the meter-type rules. No web or database code. |
| Model provider | OpenAI API (GPT-6 Sol) or Anthropic API (Claude Sonnet 5.5) | Reads the display. Receives only the photo and the fixed prompt. |
| Database | PostgreSQL 16 in production (RDS); SQLite for local runs and tests | One row per request (meter_reader_extraction_data) and per-consumer meter settings (meter_reader_profiles). |
| Photo storage | S3 bucket (SSE encryption), or a local folder, or none | Every photo, unchanged, as <YYYY-MM-DD>/<request id>.<ext>. |

In plain words The JalSoochak system sends a photo, the service asks one AI model to read it, checks the answer with its own rules, keeps a copy of everything, and replies straight away.

## 10. API reference summary

The full contract, with every field and message, is docs/INTEGRATION.md. This section summarises it.

Base paths. Every API route is served under /flowvision/v1/elm (the prefix the shared FlowVision ingress sends to this service) and, unchanged, under /meter-reader/v1 for existing clients. When METER_SERVICE_API_KEY is set, every route under either base path needs the header X-API-Key; a missing or wrong key gets HTTP 401 ERROR_09.

### Endpoints

| Method | Path (under the base path) | Purpose |
| --- | --- | --- |
| POST | /extract-reading | Read one meter photo: JSON with imageURL (the service downloads it) or imageBase64, plus details. |
| POST | /extract-reading/upload | The same, with the photo as a multipart file field image. |
| POST | /feedback | Confirm (accurate: true) or correct (actual) a returned reading, by correlationId. |
| GET | /readings/{id} | The stored record of one request, by request id or correlationId. |
| GET | /readings/{id}/image | The saved photo of one request. |
| GET / PUT | /meters/{consumerId} | Read or set a consumer's meter settings (meterTypeId, decimals, digits, billingUnit). |
| GET | /meter-types | The known meter types. |
| GET | /, /health, /ready (no base path, no key) | Greeting with the model name; liveness; readiness (database reachable within 5 s, else 503). |

### Request fields (POST /extract-reading)

| Field | Required | Meaning |
| --- | --- | --- |
| imageURL | Yes: exactly one of imageURL and imageBase64 | HTTP(S) link to the photo on the caller's image endpoint; the service downloads it. Sending neither or both gives 400 ERROR_06. |
| imageBase64 | Yes: exactly one of the two | The photo itself, base64-encoded (JPEG, PNG or WebP, at most 20 MB). |
| id | No, but always send it | The caller's request id: 1 to 128 characters (letters, digits, ., _, -, starting with a letter or digit). It is the retry key. If missing, the service generates one, and a retry can then be read and paid twice. |
| consumerId | No, but always send it | Up to 128 characters. Enables the consumer's meter settings and the previous-reading check. |
| capturedAt | No, but always send it | When the photo was taken (ISO 8601, years 2000 to 2100). A time without a zone is taken as India time (+05:30). Defaults to the arrival time. |
| previousReading | No | {value, unit, capturedAt}, value required inside it. Overrides the stored previous reading. |
| metadata | No | Any JSON object, stored with the reading. |
| any other field | No | Accepted and stored (for example village, agentId). Field names are case-sensitive: a misspelled consumerID is stored as an extra field and the consumer checks are skipped. |

Example request (illustrative values):

POST /flowvision/v1/elm/extract-reading X-API-Key: <key> { "id": "JS-EXAMPLE-0001", "imageURL": "https://images.jalsoochak.example.org/photos/0001.jpg", "consumerId": "<consumer id>", "capturedAt": "2026-09-30T10:15:00+05:30" }

Example response (abridged; values illustrative, from docs/INTEGRATION.md):

{ "id": "JS-EXAMPLE-0001", "ts": "2026-09-30T04:45:21.512Z", "responseCode": "OK", "statusCode": 200, "result": { "status": "SUCCESS", "correlationId": "5f0c2a8e-1b7d-4c1e-9d3a-2f7e0b6a9c41", "data": { "meterReading": "0019712.7", "action": "ACCEPT", "unit": "kWh", "meterBrand": "HPL PPSM 01", "decimalSource": "registry", "uncertainDigits": [], "reasons": [], "meter": {"typeId": "hpl_ppsm01", "make": "HPL", "modelCode": "PPSM 01"}, "processingTime": 19.4, "costEstimateUsd": 0.023 } } }

### Main response fields

| Field | Meaning |
| --- | --- |
| result.status / data.action | SUCCESS/ACCEPT, NEEDS_REVIEW/REVIEW, UNCLEAR/RETAKE, NOMETER/RETAKE. |
| meterReading | The reading as displayed, a string with leading zeros and the decimal point ("0019712.7"); parse it as a decimal, never a float. Returned whenever the model saw a display; ? is a digit the model could not guess; a fixed sentence when there is no number. |
| unit | kWh or kVAh on an ACCEPT; otherwise whatever unit the display showed (for example kW), or left out. |
| uncertainDigits | 1-based positions of the digits the model was unsure of (leading zeros counted, decimal point not). |
| decimalSource | Where the decimal point came from: consumer, registry, visible_point, previous_reading or model_guess. |
| reasons | List of {code, message}; the first is the main one (Appendix B). |
| retakeHint | Only on a RETAKE: the hint of the first reason that has one. |
| meter, meterBrand | Meter type id, make, model code and other printed text; meterBrand is make and model code joined. |
| previousReading | The consumer's previous trusted reading (or the one sent in the request). The new reading is compared with it only when the read passed and the units match (or the previous reading has no unit). unit or capturedAt can be null. |
| reads, notes | One summary per model call; free-text notes for people (do not branch on them). |
| model, promptVersion, processingTime, costEstimateUsd | Which model read it, the prompt version, seconds taken, estimated cost in USD. |
| correlationId | Identifies this reading for /feedback. A fresh reading gets a new one. A replay of the same id, or a same-photo result reused within 7 days, returns the original reading's correlationId. A CORRECTED_BEFORE answer gets a new one. |

Fields with no value are left out rather than sent as null. Every response under the base paths uses FlowVision's envelope (id, ts, responseCode, statusCode, and result or error). The one exception is the photo itself, which GET /readings/{id}/image returns as a plain image. The HTTP status equals statusCode unless METER_ERRORS_AS_HTTP_200=true, which sends every envelope with HTTP 200.

### Error codes

Errors carry error.errorCode, error.errorMsg and error.retryable. Retry only when retryable is true, after Retry-After when given (otherwise 5 to 10 s), up to 3 times with growing waits, always with the same id.

| Code | HTTP | Retryable | When |
| --- | --- | --- | --- |
| ERR_001 | 400 | no | Image empty, not valid base64, cannot be decoded, an unusual kind of JPEG, or too many pixels. |
| ERR_002 | 415 | no | Not a JPEG, PNG or WebP image (checked from the bytes). |
| ERROR_003 | 413 | no | Uploaded or base64 image over 20 MB, or request body over 30 MB (a photo over 20 MB at imageURL gives 400 ERROR_05). |
| ERROR_04 | 502 | per error | The model call failed or timed out (including the 100 s deadline). Retryable for timeouts, lost connections, a provider response that could not be parsed, an answer of an unexpected shape, 429 and 5xx. Not retryable for a refusal, an answer cut off at the length limit, an empty answer or one that is not a valid JSON object, an unknown model, out of credit or a rejected key. |
| ERROR_05 | 400 / 502 | 502 only | imageURL could not be downloaded: 502 if the host was down, unreachable, slow or busy (HTTP 5xx or 429); 400 if the image endpoint answered with an HTTP error such as 404, the photo is over 20 MB, or the host or address is not allowed. |
| ERROR_06 | 400 (405) | no | Missing, invalid or conflicting field, invalid JSON or id, dates outside 2000-2100, NUL, NaN or Infinity, nesting over 20 levels; 405 for a wrong method. |
| ERROR_07 | 400 | no | PUT /meters: unknown meterTypeId. |
| ERROR_08 | 404 | no | No such reading, photo, correlation id or meter settings; unknown path. |
| ERROR_09 | 401 | no | Missing or wrong X-API-Key. |
| ERROR_10 | 503 | yes (Retry-After: 10) | Busy: too many readings at once, too many large photos being received, or the request body stopped arriving. |
| ERROR_11 | 429 | only near the cap | Daily model budget used up (resumes at midnight India time); retryable with Retry-After: 60 when only readings in progress are in the way. |
| ERROR_12 | 409 | yes (Retry-After: 10) | This id is still being processed, or the same photo of this consumer is being read under another id. |
| ERROR_13 | 503 | yes (Retry-After: 10) | The database is unavailable. |
| ERROR_14 | 500 | yes | Unexpected error; quote the request id. |
| ERROR_15 | 409 | no | This id was already used for a different consumer or photo; send each new photo with a new id. |

Client rules: set the HTTP timeout to 120 s; always send your own id and reuse it on retries (a stored result comes back with header Idempotent-Replay: true, no second model call); a load-balancer 502/503/504 without the envelope is safe to retry with the same id; a photo taken after a RETAKE is a new photo and gets a new id.

## 11. The reading pipeline in detail

Figure 4 shows the order of the steps and where every reason code comes from. The order is implemented in ReadingService.extract_reading (reading_service.py), MeterReader.read (reader.py), evaluate_read and decide (checks.py).

Figure 4. The reading pipeline: the order of the checks and the code each one produces

### A. Before the model is called

- Request gate (RequestGate, before the body is read): API key (401 ERROR_09), body size over 30 MB (413 ERROR_003), photo bodies held by the worker over 150 MB (503 ERROR_10). These requests are not stored.

- Validation of the fields (400 ERROR_06) and admission: at most 2 x max_concurrent_reads (16) readings admitted per worker, otherwise an immediate 503 ERROR_10.

- Request id claim: a PROCESSING row is written before any paid work. An id that already has a result is replayed; one still in progress gets 409 ERROR_12; one used for another consumer or photo gets 409 ERROR_15; a database failure gets 503 ERROR_13.

- Read slot: wait up to 30 s for one of the 8 slots per worker, else 503 ERROR_10.

- Load the photo from base64 or a safe download of imageURL; the type is checked from the bytes (JPEG, PNG or WebP; at most 20 MB).

- Duplicate photo (same bytes, by SHA-1, and same consumer): first, a photo whose reading a person corrected comes back NEEDS_REVIEW / CORRECTED_BEFORE, whatever its age and whatever the duplicate setting; otherwise, within 7 days the earlier result is reused (header Duplicate-Of, cost 0), unless the consumer's meter settings changed since; and if the same photo of this consumer is still being read under another id, 409 ERROR_12 (Retry-After: 10; retry with the same id to get that result without a second reading).

- Budget: $0.05 per model call is reserved against the daily cap under a lock shared by all workers and instances; if it does not fit, 429 ERROR_11.

### B. The model reads the photo

The photo is prepared (EXIF rotation, longer side at most 2048 px; re-encoded as JPEG quality 92 only if rotated or resized) and sent with the fixed prompt (PROMPT_VERSION "deploy-4 (trial v2 + meter identification + best guess + tariff/prepaid/bill screens + screen label)"). The model answers in a strict JSON format: screen type, unit, each digit with whether it is legible, whether the digit count is certain, whether a decimal point is visible, the display text, meter make and model code, status (READ, UNREADABLE, NOT_CUMULATIVE_SCREEN or NO_DISPLAY), confidence, a best guess with ? filled in, and the screen label. By default the model is called once per photo (reads: 1).

### C. Checks on each answer (evaluate_read)

The checks run in the order shown in Figure 4; the first that fails decides the outcome of that read. Two details matter: the screen-label check runs before the READ check, so it also applies to answers the model could not read; and an unreadable answer whose unit is not an energy unit (kW, kVA, V, kVArh and so on) is a wrong screen (RETAKE), not a REVIEW.

When a read passes, the reading is formatted with leading zeros and the decimal point from the meter rule or, when no rule gives the decimals, from a clearly visible point (visible_point). It counts as cross-checked (digits_fixed) only when the rule supplies both the digit count and the decimals, because a digit count alone cannot catch a moved or invented decimal point.

### D. Final checks (decide)

- With several reads (reads 2 or more), any read that needs review makes the result REVIEW; otherwise any failed read makes it RETAKE; all reads passing but differing gives READS_DISAGREE.

- A consumer billingUnit that differs from the read unit gives RETAKE UNIT_MISMATCH.

- A comparable previous reading (same unit, or no unit stored): a lower reading gives BELOW_PREVIOUS; a rise of more than max_daily_increase (300) x days (at least 1 day) gives JUMP_TOO_LARGE.

- With no cross-check (not digits_fixed and no comparable previous reading), the reading is accepted only if every read's confidence is at least clear_read_confidence (0.95); otherwise NO_CROSS_CHECK.

- If the previous-reading lookup failed and the result would be ACCEPT, it becomes HISTORY_UNAVAILABLE (the paid reading is kept).

### What is returned when the reading did not pass

The response still carries the model's best reading (best_guess, else the display text), so a person has a starting value. Its decimal point comes, in this order, from the meter rule, a clearly visible point, the consumer's previous reading, or the model's own guess (decimalSource says which). When there is no number, meterReading holds a fixed sentence: "No meter display detected in the image" (NOMETER), "Not the cumulative energy reading; please retake the photo" (UNCLEAR), "Reading needs manual review" (NEEDS_REVIEW).

### After the read

The photo is saved (a failure is logged at CRITICAL and never fails the reading), the response is built with a new correlationId, and the record is stored before the response is sent (up to 5 attempts, about 7 s in all). Errors become error envelopes: a photo that cannot be prepared gives 400 ERR_001 without a model call; a model failure gives 502 ERROR_04 (its possible cost is still recorded); a database error gives 503 ERROR_13; anything else gives 500 ERROR_14.

In plain words Cheap checks come first, so a bad or repeated request costs nothing. The AI reads the photo once. Then fixed rules, in a fixed order, decide whether to accept it, ask a person or ask for a new photo, and the first rule that fails gives the reason.

## 12. Screen, label and unit rules

Meters cycle through several screens. The model is told which screen counts, and the code checks the model's work with a second rule on the label it copied (screen_label), matched by the regular expression NOT_TOTAL_LABEL in checks.py (case-insensitive). Any match is WRONG_SCREEN (RETAKE), whatever the model concluded about the screen (only a no-display answer is decided before this check).

| Kind of screen | Labels rejected (examples) | In |
| --- | --- | --- |
| Tariff or time-of-day total | T1 to T8 (also T01, T.1, T-1, kWhT1), TOD, TOU, TZ, TARIFF, RATE 1, ZONE, Z1, Roman TI to TVIII, TR1, PEAK, OFF PEAK, OBIS 1.8.1 to 1.8.9 | main and latest |
| Bill history | BILL, 01 Bill, a bare 01 to 09, HIST, PREV, LAST | main and latest |
| Maximum demand | MD, MDkW, MAX DEMAND, DEMAND | main and latest |
| Prepaid balance or money | INR, Rs, ₹, BAL, CREDIT, LTR | main and latest |
| Export register | OBIS 2.8.x | main and latest |
| Export register | EXPORT, EXP, EXPORTED (also Exp. kWh, kWhEXP) | latest only |
| History register | H alone, H1 to H12 (also H 1, H-1) | latest only |
| Billing register | B1 to B12 (also B 1, B-1) | latest only |

Labels that still pass (tested): CUM kWh, kWh, kVAh, Cum. kVAh, TOTAL kWh, IMPORT kWh, NET kWh, OBIS 1.8.0, the unit's own h (kW h, kW·h, kVA h), and the phase indicator R Y B including a lone B. Ranges stop at 12 for H and B and at 1-8 for T, so H13, B13, T9 and T0 are not rejected. H01 to H09 and B01 to B09 are already rejected by the deployed code through the bare 01 to 09 bill-index rule.

Units. The unit is normalised first (spaces, ·, ., - and _ removed, case ignored), so KWH, CUM kWh and kW h all mean kWh. Only kWh and kVAh are accepted. A read in kW or kVA (maximum demand), V, A, Hz, PF, money units or kVArh (reactive energy: a cumulative register, but not the target) is the wrong screen. A text naming two units, such as kWh/kVAh, is not accepted. A clear read with no unit at all is UNIT_NOT_VISIBLE (RETAKE), because these meters show both kWh and kVAh.

Known side effect. Genus SAKSHAM-345 meters show T1 CUM kWh on their normal total screen; T1 matches the tariff rule, so these photos are retakes. A change (do not treat T1 as a tariff label when CUM is on the same screen) is proposed, not made. To change a label rule, edit NOT_TOTAL_LABEL and add examples, including labels that must still pass, to test_label_of_a_non_total_screen_overrides_the_model in tests/test_checks.py.

In plain words If the small text next to the number says the screen is a tariff, an old bill, maximum demand, a balance or (latest code) a history, billing or export register, the photo is retaken. A unit other than kWh or kVAh also means the wrong screen.

## 13. Meter types and the decimal point

The decimal point on these displays is a tiny dot that a model can miss or invent. So the service takes the decimal position, and where possible the digit count, from the meter type: a known model recognised from the model code printed on the meter, never from the brand alone (L&T / Schneider make both the ER300, with no decimal point, and the EM101+, with one decimal digit). Only types confirmed on the project's own photos are listed.

| Type id | Meter | Digits | Decimals | Example format |
| --- | --- | --- | --- | --- |
| hpl_ppsm01 | HPL PPSM 01 three-phase smart meter (PSPCL, Punjab) | 8 | 1 | 0019712.7 |
| hpl_ppem26 | HPL PPEM 26 (PSPCL, Punjab) | 7 | 1 | 999999.9 format |
| lt_er300 | L&T / Schneider ER300 static meter (APDCL, Assam) | 8 | 0 | 00007696 |
| lt_em101 | L&T / Schneider EM101+ single-phase (APDCL, Assam) | varies | 1 | 2344.7 |
| genus_saksham345 | Genus SAKSHAM-345 (APDCL, Assam) | 8 | 1 | 0032740.9 |
| visiontek_37tm | Visiontek 37TM (Punjab) | 8 | 1 | 9999999.9 format |
| avon_asem306 | Avon ASEM-306 (Punjab) | 8 | 1 | 9999999.9 format |
| aew_aplsm346 | AEW APL-sm346 (Punjab) | not fixed | 1 | 9999999.9 format |

Six of the eight types have a fixed digit count, so for them a dropped digit is caught (DIGIT_COUNT_MISMATCH). The patterns are matched case-insensitively against the model code, make and faceplate text the model copied; a type is used only when exactly one type matches.

### How the rule for a photo is built

- Each per-consumer setting (PUT /meters/{consumerId}: meterTypeId, decimals, digits) wins.

- A setting left empty comes from the consumer's chosen meter type, else from the type identified in the photo, so a partial setting never drops the digit-count check.

- With no decimals known anywhere, only a clearly visible decimal point counts (visible_point); otherwise DECIMAL_UNCERTAIN (REVIEW).

- An ACCEPT can only have decimalSource consumer, registry or visible_point; previous_reading and model_guess appear only on best-guess readings of a REVIEW or RETAKE.

Adding a type: add it to src/service/vision/meter_types.py, or without a code change in a JSON file named by METER_TYPES_FILE (fields id, description, decimals, digits, patterns). Use only formats confirmed on real photos. GET /meter-types lists the types in use.

## 14. Previous readings, feedback and corrections

### The trusted previous reading

Each new reading is compared with the consumer's latest trusted reading taken at or before this photo's capture time (newest first, same unit or no unit stored). Trusted readings are:

- SUCCESS readings not marked wrong;

- a SUCCESS or NEEDS_REVIEW reading confirmed with accurate: true (it must be a full number);

- any reading sent as actual, whether a correction or a confirmation (for a RETAKE result, only an explicit actual counts).

Reused answers (duplicates) never count, rows without a capture time are skipped, and a stored value that cannot be parsed as a reading is skipped so it cannot switch the check off. A previousReading sent in the request overrides the stored one.

### Feedback

POST /flowvision/v1/elm/feedback {"correlationId": "5f0c2a8e-1b7d-4c1e-9d3a-2f7e0b6a9c41", "data": {"accurate": false, "actual": "0019721.7"}}

- Send accurate, actual, or both; at least one is required. Send actual as a string so leading zeros are kept (digits with an optional decimal point, else 400 ERROR_06).

- A REVIEW reading with a ?, or with no number, cannot be confirmed: send the correct value as actual.

- The answer is result.status SUBMITTED; an unknown correlationId gives 404 ERROR_08.

- Feedback is stored on the record (extracted_reading_accurate, actual_reading, feedback_timestamp).

### What a correction does

- A correction sent with actual becomes the consumer's trusted previous reading, so every later photo of that consumer is checked against it (BELOW_PREVIOUS, JUMP_TOO_LARGE) and can avoid NO_CROSS_CHECK. A correction sent only as accurate: false just marks the reading wrong: it stops being trusted, and nothing replaces it.

- A correction is accurate: false, or an actual that differs from the returned reading; an actual equal to the reading is a confirmation.

- The same photo (same bytes, same consumer) sent again after a correction is never read or accepted again: it comes back NEEDS_REVIEW / CORRECTED_BEFORE with the earlier model reading, a new correlationId, header Duplicate-Of and cost 0. Feedback on it should use the original reading's correlationId.

- Feedback does not change the prompt, the meter types or the thresholds automatically; those are changed by people, after evaluation.

### Per-consumer meter settings

PUT /meters/{consumerId} with meterTypeId (one of GET /meter-types, else 400 ERROR_07), decimals (0-3), digits (1-12) and billingUnit (kWh or kVAh). A PUT replaces all four settings: a field left out is cleared; the same settings again change nothing. With billingUnit set, a clear reading of the other unit is RETAKE UNIT_MISMATCH; a hard-to-read photo still goes to review whatever its unit, so the reviewer should check unit too.

In plain words Every confirmed or corrected reading teaches the service what that consumer's meter said last time, and the next photo is checked against it. Telling the service which meter model a consumer has lets more readings be accepted automatically.

## 15. Protections and limits

None of these limits stops the service by itself; each acts only in the situation it guards against. "Config only" means the value is set in src/conf/config.yaml and has no environment variable.

| Protection | Setting | Default | What happens |
| --- | --- | --- | --- |
| Safe retries (request id) | always on | - | Same id returns the stored result (Idempotent-Replay: true); in progress 409 ERROR_12; other consumer or photo 409 ERROR_15. |
| Same photo reused | METER_DUPLICATE_PHOTO_HOURS | 168 (7 days) | Earlier result, no model call, cost 0. 0 turns reuse off (a corrected photo still goes to review). |
| Daily model spend | METER_DAILY_BUDGET_USD | 25 (USD per India-time day) | 429 ERROR_11 until midnight India time; CRITICAL log. none = no cap (on purpose only). |
| Reservation per model call | limits.reserve_per_call_usd (config only) | 0.05 | Reserved before the call, replaced by the real cost; the costliest real call seen was $0.043. |
| Time per reading | METER_REQUEST_DEADLINE | 100 s | 502 ERROR_04, retryable. Gunicorn timeouts are derived from it. |
| Model retries | METER_OPENAI_ATTEMPTS; claude.max_attempts (config only) | 2 attempts | One retry, only for errors the provider did not bill (429, 500, 502, 503, and 529 for Anthropic) or a connection never opened; never after a timeout or dropped connection. |
| Readings at once | METER_MAX_CONCURRENT_READS, METER_BUSY_WAIT_SECONDS | 8 per worker, 30 s | Up to 8 more wait for a slot, then 503 ERROR_10; beyond that, 503 at once. |
| Request body | METER_MAX_BODY_BYTES | 30 MB | 413 ERROR_003, before the body is read. |
| Photo bodies held at once | METER_MAX_INFLIGHT_BODY_BYTES | 150 MB per worker | 503 ERROR_10 (retryable). |
| Image size | file_size_limit (config only) | 20 MB | 413 ERROR_003 (uploaded or base64 image); 400 ERROR_05 for an imageURL photo over 20 MB. |
| Image pixels | reader.max_jpeg_pixels, reader.max_image_pixels (config only) | 250 MP (single-scan JPEG), 16 MP (PNG, WebP, other JPEG) | 400 ERR_001, checked from the header before decoding; one full-size decode at a time per worker. |
| Image size sent to the model | reader.max_image_side (config only) | 2048 px | The longer side is shrunk to this. |
| Minimum confidence | METER_MIN_CONFIDENCE | 0.90 | Below it: REVIEW LOW_CONFIDENCE. |
| Clear-read confidence | METER_CLEAR_READ_CONFIDENCE | 0.95 | No cross-check and below it: REVIEW NO_CROSS_CHECK. |
| Largest daily increase | METER_MAX_DAILY_INCREASE | 300 per day | Above it: REVIEW JUMP_TOO_LARGE. |
| Reads per photo | METER_READS | 1 | 2 = read twice, accept only if both agree (doubles the cost). |
| Client authentication | METER_SERVICE_API_KEY | unset (production refuses) | 401 ERROR_09. Several comma-separated keys allow rotation. |
| Image download hosts | METER_IMAGE_URL_ALLOWED_HOSTS | any public host (production requires a value) | Other hosts 400 ERROR_05. Private addresses refused unless METER_ALLOW_PRIVATE_IMAGE_URLS=true; cloud-metadata and link-local addresses always refused. |
| Download time | image_download.timeout_seconds (config only) | 30 s | 502 ERROR_05, retryable; also capped by the request deadline. |

Image downloads have further safeguards: links with a user name or password are refused; at most 3 redirects, each checked; the socket connects only to the address that was checked (no DNS rebinding); no proxy is used; credential headers (METER_IMAGE_URL_HEADERS) go only to allowed hosts over HTTPS (or plain HTTP to a private address when private hosts are allowed) and are dropped on a cross-origin redirect; links are logged and stored without query string and fragment.

Request contents are also limited: dates outside 2000-2100, the NUL character, NaN or Infinity, and nesting deeper than 20 levels are refused with 400 ERROR_06.

## 16. Model providers and configuration

|  | OpenAI | Anthropic |
| --- | --- | --- |
| METER_PROVIDER | openai (default in config.yaml) | claude |
| Model (default) | gpt-6-sol | claude-sonnet-5-5 |
| Key | OPENAI_API_KEY | ANTHROPIC_API_KEY (plus ANTHROPIC_WORKSPACE_ID only for a key not scoped to a workspace) |
| Effort | reasoning_effort high; image detail high | effort high (low, medium, high, xhigh, max, or none); adaptive thinking, which bills as output |
| Client | Standard-library HTTP client (no OpenAI SDK), Chat Completions with a strict JSON schema | Anthropic Python SDK (its own retries off), structured JSON output |
| Retried statuses | 429, 500, 502, 503 (not insufficient_quota) | 429, 500, 502, 503, 529 |
| Photo size limit | 20 MB (service limit) | Also Anthropic's 5 MB per image as base64: larger photos re-encoded as JPEG at quality 90, 80, then 70 |
| Price used for cost and budget | $2.0 input / $10.0 output per million tokens | $2.0 input / $10.0 output per million tokens |
| Measured 21 Oct 2026 | $0.0223 per photo, median 23.5 s (100 photos) | $0.0137 per photo, median 4.2 s (100 photos); median $0.0136 and 4.2 s on 268 |

### Switching provider

- Set METER_PROVIDER=claude (or openai) and that provider's key, as a secret.

- Remove METER_MODEL, or set it to a model of the chosen provider: the service refuses to start if the model name does not match the provider (a Claude model name must start with claude).

- If METER_REASONING_EFFORT is set, it must be valid for the provider (for Claude: low to max, or none).

- Restart the service (on EC2, systemctl restart, not reload). The startup log line names the model and provider, and GET / returns the model.

- For Claude, have the Anthropic data-sharing approval in place before real photos are sent, and set a monthly budget and alerts on the Anthropic account.

There is no fallback: only the chosen model reads a photo. If Claude declines a photo, the reading fails with ERROR_04.

### Environment variables (names and defaults)

Most settings are in src/conf/config.yaml (or the file named by CONFIG_PATH), most with the environment variable that overrides them. A few exist only as environment variables: METER_ENV, the database pool and timeout settings (METER_DB_POOL_SIZE, METER_DB_MAX_OVERFLOW, METER_DB_POOL_TIMEOUT, METER_DB_POOL_RECYCLE, METER_DB_NULL_POOL, METER_DB_CONNECT_TIMEOUT, METER_DB_STATEMENT_TIMEOUT_MS, METER_DB_TCP_USER_TIMEOUT_MS; METER_DB_URL overrides database.url in config.yaml) and METER_LOG_FORMAT / METER_LOG_TO_FILES / METER_LOG_RETENTION_DAYS. .env.example lists the main variables, and gunicorn.conf.py reads the gunicorn ones (WORKERS, BIND, TIMEOUT, GRACEFUL_TIMEOUT, KEEPALIVE, MAX_REQUESTS, MAX_REQUESTS_JITTER, FORWARDED_ALLOW_IPS, LOG_LEVEL). Secrets are never placed in the code or the repository.

| Variable | Default | Purpose |
| --- | --- | --- |
| METER_PROVIDER | openai | openai or claude |
| OPENAI_API_KEY / ANTHROPIC_API_KEY | none (required for the provider in use) | Provider key |
| ANTHROPIC_WORKSPACE_ID | empty | Workspace for a key not scoped to one |
| OPENAI_BASE_URL / ANTHROPIC_BASE_URL | https://api.openai.com/v1 / https://api.anthropic.com | Provider API address |
| METER_MODEL | gpt-6-sol / claude-sonnet-5-5 | Model of the chosen provider |
| METER_REASONING_EFFORT | high | OpenAI reasoning effort or Claude effort; none = model default |
| METER_IMAGE_DETAIL | high | OpenAI image detail |
| METER_REQUEST_TIMEOUT | 90 | Longest wait for one model answer (s) |
| METER_OPENAI_ATTEMPTS | 2 | OpenAI attempts in total (1 = no retry) |
| METER_READS | 1 | Model calls per photo (must agree) |
| METER_MIN_CONFIDENCE / METER_CLEAR_READ_CONFIDENCE | 0.90 / 0.95 | Check thresholds |
| METER_MAX_DAILY_INCREASE | 300 | Largest plausible increase per day |
| METER_TYPES_FILE | none | Extra meter types (JSON) |
| METER_ENV | development | production turns on the startup checks, JSON logs and docs off |
| METER_SERVICE_API_KEY | unset | Accepted X-API-Key values, comma-separated |
| METER_DB_URL | sqlite:///data/meter_reader.db | Database (PostgreSQL in production) |
| METER_IMAGE_DIR | data/images | Photo folder, s3://bucket/prefix, or none |
| METER_IMAGE_URL_ALLOWED_HOSTS | any public host | Hosts photos may be downloaded from, or * |
| METER_IMAGE_URL_HEADERS | {} | Credential headers for the allowed hosts (JSON) |
| METER_ALLOW_PRIVATE_IMAGE_URLS | false | Allow image hosts on private addresses |
| METER_DAILY_BUDGET_USD | 25 | Daily model spend cap; none = no cap |
| METER_DUPLICATE_PHOTO_HOURS | 168 | Same-photo reuse window; 0 = off |
| METER_REQUEST_DEADLINE | 100 | Seconds per reading |
| METER_MAX_CONCURRENT_READS / METER_BUSY_WAIT_SECONDS | 8 / 30 | Readings at once per worker; wait for a slot (s) |
| METER_MAX_BODY_BYTES / METER_MAX_INFLIGHT_BODY_BYTES | 31457280 / 157286400 | Largest body; photo bodies held at once per worker |
| METER_DOCS_ENABLED | on in development, off in production | /docs, /redoc, /openapi.json |
| METER_ERRORS_AS_HTTP_200 | false | FlowVision-style HTTP 200 for every envelope |
| METER_NAIVE_TIME_OFFSET | +05:30 | Time zone for times sent without one |
| METER_LOG_LEVEL / METER_LOG_FORMAT | info / json in production, else text | Logging |
| METER_LOG_TO_FILES / METER_LOG_RETENTION_DAYS | on in development, off in production and in the image / 30 | Daily log files |
| METER_DB_POOL_SIZE / METER_DB_MAX_OVERFLOW | 5 / 10 | Database connections per worker |
| METER_DB_POOL_TIMEOUT / METER_DB_POOL_RECYCLE | 10 / 1800 | Seconds |
| METER_DB_CONNECT_TIMEOUT | 5 | Seconds (PostgreSQL) |
| METER_DB_STATEMENT_TIMEOUT_MS / METER_DB_TCP_USER_TIMEOUT_MS | 15000 / 15000 | Slow statements and lost connections |
| WORKERS / BIND | 2 / 0.0.0.0:8000 | Gunicorn workers and address |
| TIMEOUT / GRACEFUL_TIMEOUT / KEEPALIVE | deadline + 60 / deadline + 20 / 130 | Gunicorn timeouts (s) |

Settings without an environment variable: file_size_limit, reader.max_image_side, reader.max_image_pixels, reader.max_jpeg_pixels, limits.reserve_per_call_usd, image_download.timeout_seconds, the prices, claude.max_attempts, basepaths, the log file paths (logs) and app_server.

## 17. Storage, database and privacy

### Database tables

The service creates missing tables, columns and indexes itself on every start (on PostgreSQL under an advisory lock, so several workers can start at once). meter_reader_db_ddl.sql holds the same schema for DBAs; if used, it must run as the service's database user. The schema only ever gains columns and indexes.

| Table | Key | Important columns |
| --- | --- | --- |
| meter_reader_extraction_data (one row per request) | request_id | correlation_id, consumer_id, image_url (without query), image_sha1, request_fingerprint, duplicate_of, image_path, metadata, request_timestamp, captured_at, meter_reading_status (SUCCESS, UNCLEAR, NOMETER, NEEDS_REVIEW, ERROR, PROCESSING), meter_reading, unit, meter_type, reasons, result, model, processing_time, cost_usd, response_timestamp, extracted_reading_accurate, actual_reading, feedback_timestamp, request_json (without image data), response_json |
| meter_reader_profiles (per-consumer settings) | consumer_id | meter_type_id, decimals, digits, billing_unit, updated_at |

Indexes support the previous-reading lookup (consumer_id, captured_at) and lookups by consumer, correlation id, photo hash and time. Requests refused before they are read (wrong key, body over 30 MB, photo-body limit, invalid JSON or an invalid id) are not stored; other refused requests that carry a valid id are stored with status ERROR.

### Photos

| Where | How |
| --- | --- |
| Production | S3: s3://bucket/prefix, uploads with ServerSideEncryption="AES256"; the bucket blocks public access and has a lifecycle rule for the agreed retention period (for example 365 days). |
| Local | data/images/<YYYY-MM-DD>/<request id>.<jpg¦png¦webp> |
| Not kept | METER_IMAGE_DIR=none |

Photos are saved unchanged and the file name holds no consumer details; the database links photo, consumer and reading. GET /readings/{id}/image returns a photo and needs the API key.

### Logs

- Every record goes to stdout, one line each, JSON in production (time, level, logger, message). Daily files under logs/ are written by default only in development (METER_LOG_TO_FILES turns them on or off).

- The logs contain each request (with consumer id, metadata and extra fields, without imageBase64, with the URL redacted), each response, feedback and meter-setting changes.

- The photo never appears in the logs at any level (the Anthropic SDK's debug logging, which would include it, is suppressed); API keys never appear in logs, responses or the database; access-log lines have their query string removed; the database password is hidden in the startup line; unexpected errors that reach the service's last-resort handler are logged with only the exception type and the first 300 characters of the message, so a validation error cannot carry the request (and the photo) into the log.

- CRITICAL lines (for alarms): daily budget reached; provider out of credit or key rejected (and, with Claude, an unknown model); a result that could not be stored; a photo that could not be saved; a gunicorn worker timeout.

### Data sent to the model provider

One message with the fixed prompt and the photo (rotated, shrunk to 2048 px and re-encoded only when needed; for Claude, re-encoded further if over 5 MB as base64). A photo that needs no rotating or shrinking is sent byte for byte as received, so any metadata inside the file (for example EXIF data from the phone) goes with it; a re-encoded photo carries none. Never consumer ids or other request details.

In plain words The database knows which photo belongs to which consumer; the photo files and the AI provider do not. Logs do contain consumer ids, so they need a retention period like the photos.

## 18. Deployment and operations

### Technology

| Part | Version or choice |
| --- | --- |
| Language and runtime | Python 3.12; Docker base image python:3.12-slim-bookworm |
| Web | FastAPI 0.142.2, served by gunicorn 26.2.0 with uvicorn_worker.UvicornWorker (uvicorn 0.54.0) |
| Database access | SQLAlchemy 2.1.1, psycopg2-binary 2.9.13; PostgreSQL 16 |
| Images and storage | Pillow 12.3.0; boto3 1.43.105 for S3 |
| Model clients | Anthropic SDK 1.15.0 for Claude; a standard-library client for OpenAI (no OpenAI SDK) |
| Dependencies | Pinned with hashes in requirements.lock, installed with pip install --require-hashes |

### The Docker image

Multi-stage Dockerfile (builder, base, test, and runtime, the default target). It runs as the non-root user 10001:10001, copies only src and gunicorn.conf.py, exposes port 8000 and starts gunicorn -c gunicorn.conf.py routes:app. A container health check calls /health every 30 s. .env files and docs never enter the image. No OpenCV or PyTorch: the reading is done by the provider's API.

### Ways to run it

| Option | Summary |
| --- | --- |
| ECS Fargate (recommended) | Task with 1 vCPU and 4 GB, port 8000, stopTimeout 120 s, container health check on /health; service with 2 tasks across two subnets behind the ALB (health-check grace 60 s, deregistration delay 120 s). A new version is a new image tag in the task definition; ECS rolls tasks one at a time. |
| EC2 with systemd | Dedicated user, Python 3.12 virtual environment, settings in /etc/meter-reader.env (mode 600), unit meter_reader.service (gunicorn, Restart=always, sandboxed, TimeoutStopSec=130). Code update: git pull then systemctl reload (no request refused); settings change: systemctl restart. |
| Kubernetes | Manifests live in a separate repository (ADR-001). Liveness probe /health, readiness probe /ready; runAsNonRoot; WORKERS=1 per pod and scale with replicas; terminationGracePeriodSeconds at least 130; ingress prefix /flowvision/v1/elm with no rewrite, read and send timeouts 120 s and body size 30 MB. |
| Docker Compose (one machine) | docker compose up --build -d: the service in production mode plus PostgreSQL 16, on 127.0.0.1:8000; then curl localhost:8000/ready. |

AWS resources: RDS PostgreSQL 16 (db.t4g.small is enough for a pilot; backups, encryption, private subnets); an S3 bucket (public access blocked, SSE-S3, lifecycle rule); Secrets Manager for the provider key, METER_SERVICE_API_KEY and METER_DB_URL; an IAM role allowed only s3:PutObject and s3:GetObject on the photo prefix; an HTTPS ALB with idle timeout 120 s and health check GET /health; security groups (service port 8000 from the ALB only); a CloudWatch log group with a retention period.

### Continuous integration

.github/workflows/image-build-elm-ocr.yaml ("Build and publish ELM OCR image") runs on every pull request and every push to main that touches services/elm-ocr/** or the workflow, and can be started by hand or called from another workflow. The test job runs the whole test suite on Python 3.12 against PostgreSQL 16. On a push to main, a manual run or a call from another workflow (never on pull requests) the build-and-push job then builds the image and pushes it to Amazon ECR as meter-reader:<branch>-<commit sha>. Deploys stay manual.

### Health checks

| Endpoint | Meaning | Use |
| --- | --- | --- |
| GET /health | Liveness: {"status": "ok"}, no dependencies; answers even when every read slot is busy | Load balancer and container health checks; Kubernetes liveness |
| GET /ready | Readiness: database answers within 5 s; returns readsInProgress and readSlots; 503 {"status": "unavailable", "database": "unreachable"} otherwise | Kubernetes readiness; alarm on it |

### Production mode

With METER_ENV=production the service refuses to start until these are set, and the error names what is missing: METER_SERVICE_API_KEY (at least 16 characters, not a placeholder), METER_DB_URL (PostgreSQL), METER_IMAGE_DIR, METER_IMAGE_URL_ALLOWED_HOSTS, and a daily budget (already 25 in config.yaml). METER_ALLOW_PRIVATE_IMAGE_URLS=true and METER_IMAGE_URL_HEADERS both need a real host list (not *). The provider key is required in every mode.

### Go-live checklist (from DEPLOY_AWS.md and INTEGRATION.md)

- RDS, S3 bucket (public access blocked, lifecycle rule), secrets, IAM role and security groups created.

- ALB on HTTPS, idle timeout 120 s, health check /health.

- METER_ENV=production and all required variables set; the $25 daily cap fits the expected volume.

- A monthly budget and alerts on the provider account (OpenAI, or Anthropic with Claude).

- Provider key rotated and the key used during development revoked.

- Smoke test passes: /health, /ready, and one real reading with scripts/run.py and a new consumer id.

- CloudWatch alarms exist and notify someone.

- JalSoochak has docs/INTEGRATION.md, the URL and the API key; client timeout 120 s; retries reuse the id; billing only on ACCEPT or confirmed readings; RETAKE shows the hint and sends a new id.

- JalSoochak's image host is in METER_IMAGE_URL_ALLOWED_HOSTS and a real imageURL works.

- Someone owns the REVIEW queue and sends /feedback.

### Monitoring

Suggested CloudWatch alarms: ALB 5XX count above a few per 5 minutes; ALB response time p95 above 60 s; unhealthy hosts above 0; a log filter on { $.level = "CRITICAL" }; a log filter on *out of memory*; RDS CPU, free storage and connections. A daily summary query: SELECT meter_reading_status, count(*), sum(cost_usd) FROM meter_reader_extraction_data WHERE request_timestamp > now() - interval '1 day' GROUP BY 1;

### Capacity

One task with 2 workers x 8 slots handles about 0.8 photos per second (about 69,000 a day) with GPT-6 Sol, and about two to four times that with Claude; the provider's own rate limits usually bind first. Measured in Docker with 2 workers, a burst of 64 photos of 3.6 MB peaked at about 0.7 GB; 2 GB held in the tests and a 4 GB task is the safety margin. Each worker opens up to 15 database connections; keep tasks x workers x 15 under the RDS connection limit. This is processing capacity only: the default $25 daily budget stops model calls after about 1,100 photos a day at GPT-6 Sol's measured $0.0223, or about 1,800 at Claude's $0.0137, so raise METER_DAILY_BUDGET_USD for higher volumes.

### Routine operations

- Rotate the service API key: set METER_SERVICE_API_KEY=new,old, deploy, let JalSoochak switch, then set new alone.

- Rotate the provider key or database password: update the secret and redeploy or restart.

- Switch provider: section 16.

- Data retention: S3 lifecycle rule for photos; delete old database rows on the agreed schedule; CloudWatch log retention.

- Rollback: redeploy the previous image tag or git revision; the schema only gains columns and indexes, so older versions keep working. Some retries of ids across versions get 409 ERROR_15 (no money spent; the stored result stays readable with GET /readings/{id}). Let pending retries finish, or pause them, before rolling back.

## 19. Testing and evaluation

### Automated tests

python -m pytest -q runs 261 tests with a fake model, so they make no API calls and cost nothing; 3 of them run only on PostgreSQL (METER_TEST_DB_URL). The Claude tests replace Anthropic's API with a scripted fake at the HTTP level, under the real SDK. Tests never see the local .env or METER_* settings.

| File | Tests | What it covers |
| --- | --- | --- |
| test_api.py | 16 | Envelope, status mapping, review and confirm flow, errors, upload, meter settings, feedback and history, photo save and fetch, API key, legacy base path |
| test_checks.py | 22 | Meter-type rules, retake vs review, units, non-total screen labels, clear-read rule, decimal sources, decide |
| test_claude.py | 33 | The Claude provider: prompt and schema, workspace header, effort, retries, timeouts, no photo in debug logs, images over 5 MB |
| test_integration.py | 5 | The caller's side: details stored with each reading, photos pulled from its image endpoint |
| test_production.py | 41 | Production behaviour: auth, error envelope, idempotency, limits, health, time zones, storage, OpenAI retries |
| test_reader.py | 12 | Accepted reading and cost, two reads must agree, consumer profile, billing unit, previous reading and trust rules |
| test_system_regressions.py | 22 | One test per bug found by the live system test of 2026-10-01 |
| test_round2_regressions.py | 110 | One test per root cause found by the second live system test (2026-10-05) |

Counts include parametrised cases. Before relaxing a rule that looks over-cautious, search the two regression files for it.

### Measuring accuracy with real model calls

This needs a hand-checked photo set (kept outside the repository, because the photos are private): a CSV with image_id, truth_kind (reading, not_cumulative or unreadable) and truth_display, and a folder of photos named <image_id>.jpg.

python scripts/evaluate.py --review review.csv --images photos/ --yes --limit 10 # real API, 10 photos python scripts/evaluate.py --review review.csv --images photos/ --yes # all; data/evaluation.jsonl python scripts/replay.py data/evaluation.jsonl --images photos/ # re-score saved answers (free)

- evaluate.py runs the full reader with the configured provider, costs money (--yes confirms), saves the model's full answers and resumes where it stopped. Each photo is judged on its own, without history. Options: --limit, --reads, --workers (default 4), --fresh, --out.

- replay.py feeds saved answers through the current checks, with optional --min-confidence and --clear-read-confidence, so a change to checks, meter types or thresholds is measured for free. It cannot measure prompt changes.

- scripts/run.py sends one photo to a running service and prints the reading and the stored record (used for the smoke test).

### How to evaluate a change

- Add or update unit tests for the rule (for label rules, include labels that must still pass) and run python -m pytest -q.

- For a change to checks, meter types or thresholds: run replay.py on saved evaluation answers and compare the outcomes that changed.

- For a change to the prompt or the model: update PROMPT_VERSION (prompt changes), then run evaluate.py on a hand-checked set; compare wrong accepts first, then right readings returned, retakes, cost and time.

- To confirm the accuracy target (at least 95% of ACCEPT readings right) before going live, use a fresh hand-checked set large enough to give about 60 ACCEPT readings with zero errors.

- Open a pull request: CI runs the tests on PostgreSQL; after merge to main it builds and pushes the image; deployment is a separate, manual step.

## 20. Codebase guide

| Path | What it does |
| --- | --- |
| src/run.py, src/routes.py | Development server (uvicorn); routes:app is the object gunicorn serves |
| src/app_factory.py | Builds the FastAPI app: RequestGate (key, size and body limits before the body is read), error envelope, health endpoints, all routes under each base path |
| src/conf/ | config.yaml (the settings, naming the environment variable that overrides each; a few, such as METER_ENV, the database pool and timeout settings and the log-file settings, exist only as environment variables), config.py (loading, check_production), logging.py |
| src/error/error.py | Error codes ERR_001 ... ERROR_15 and CustomHTTPException with retryable |
| src/models/models.py | Request and response models (same envelope and field names as FlowVision's BFM service) |
| src/validation/validators.py | ImageValidator: size and real file type |
| src/service/api/reading_service.py | One request from start to finish; feedback; meter settings |
| src/service/api/metadata_service.py | Storage logic: claim, claim_photo, reserve, store_extraction, store_feedback, last_trusted, profiles |
| src/service/api/database.py | SQLAlchemy engine, the two tables, schema creation on start |
| src/service/api/guards.py | DailyBudget and ReadSlots |
| src/service/api/image_store.py | LocalImageStore, S3ImageStore |
| src/service/vision/reader.py | MeterReader.read: prepare image, call the model, apply checks, compute cost |
| src/service/vision/prompt.py | PROMPT, the JSON SCHEMA, PROMPT_VERSION |
| src/service/vision/openai_client.py, claude_client.py | The two provider clients, with the same contract and ModelError |
| src/service/vision/settings.py | ReaderSettings: the reader's settings and their defaults |
| src/service/vision/checks.py | REASONS, NOT_TOTAL_LABEL, evaluate_read, candidate, decide |
| src/service/vision/meter_types.py | The 8 built-in meter types and MeterTypeRegistry |
| src/service/vision/images.py, net.py | Image preparation and safe URL download; connections with an overall time limit and address checks |
| scripts/ | run.py (send a photo), evaluate.py (paid evaluation), replay.py (free re-scoring) |
| tests/ | pytest suite with a fake model (section 19) |
| docs/ | INTEGRATION.md (API contract), DEPLOY_AWS.md (deployment and operations) |
| Dockerfile, gunicorn.conf.py, docker-compose.yml, meter_reader.service | Image, server settings, local production-like stack, systemd unit for EC2 |
| meter_reader_db_ddl.sql, requirements.txt, requirements.lock, requirements-dev.txt | Schema for DBAs; direct and hash-pinned dependencies; test tools |

### Following one request through the code

- RequestGate in app_factory.py: API key, body size and photo-body limit, before the body is read.

- The route extract_reading (or extract_reading_upload) admits the request and hands it to the service in a worker thread.

- ReadingService.extract_reading: claim the id, take a read slot, load the photo, claim the photo (duplicate check), reserve the budget, call the reader, save the photo, build the response, store the record.

- MeterReader.read: prepare the image, load the consumer profile, call the model, then evaluate_read for each answer, last_trusted for the previous reading, decide, and candidate for a best-guess reading.

- Back in reading_service.py: the status, action and response, stored before the reply is sent.

## 21. Decisions and open items for the team

Owners are not named in the repository; the team should assign one to each item.

| # | Item | Current state | Decision or action needed |
| --- | --- | --- | --- |
| 1 | Provider in production | Claude Sonnet 5.5 chosen on 21 Oct 2026; code default still openai | Set METER_PROVIDER=claude and ANTHROPIC_API_KEY in the deployment; remove or match METER_MODEL |
| 2 | Anthropic data-sharing approval | README: needed before real photos are sent to Anthropic; status not recorded. The 21 October evaluation sent the 268 hand-checked field photos to Anthropic | Confirm the approval (and record it), and confirm whether the evaluation's use was covered |
| 3 | Merge elm-optimized into main | H, B and EXPORT label rejection only on elm-optimized; not yet measured on the photo set | Merge, deploy, and re-run the evaluation |
| 4 | T1 + CUM rule (Genus SAKSHAM-345) | Proposed, not made; would recover 4 retakes in 268 | Decide; if yes, change NOT_TOTAL_LABEL with tests |
| 5 | REVIEW with no usable number | 5 of 268; making these a RETAKE is a proposed option, not made | Decide |
| 6 | Owner of the REVIEW queue | Required by the go-live checklist | Name a person or team that confirms or corrects every REVIEW through /feedback |
| 7 | Daily budget | $25 per day (about 1,100 photos at GPT-6 Sol's measured cost, about 1,800 with Claude) | Check against the expected volume; set a monthly budget on the provider account |
| 8 | Retention periods | Examples in the docs: photos 365 days, database rows 2 years, logs 90 days; not agreed | Agree the periods and configure them |
| 9 | Image host and credentials | Needed for METER_IMAGE_URL_ALLOWED_HOSTS and, if required, METER_IMAGE_URL_HEADERS | JalSoochak to provide |
| 10 | Accuracy target (at least 95% of ACCEPT readings right) | 9 ACCEPT readings with Claude Sonnet 5.5 on the 268 photos (21 Oct; all correct); about 60 with zero errors are needed | Plan a larger fresh hand-checked evaluation |
| 11 | Per-consumer meter settings | Setting the meter type lets more readings be accepted automatically | Decide who fills PUT /meters/{consumerId} |
| 12 | Provider key rotation | The OpenAI key used during development must be rotated and revoked before the pilot | Rotate and revoke |
| 13 | Out-of-date notes | Some README and code comments still say Claude has not been evaluated | Refresh them in a later change |
| 14 | ER300 meter format (8 digits, no decimal) | One of the two example values listed as evidence for this type (00018103) was seen on 21 October as a frozen past-bill value (the same number on 6 photos of one consumer over 10 days) | Re-confirm the format on fresh total-screen photos |

# Appendix A: Glossary

| Term | Meaning |
| --- | --- |
| ACCEPT (SUCCESS) | The reading passed every check and can be used. |
| API | The agreed way one computer system asks another for something; here JalSoochak sends a photo and gets a reading back. |
| API key (X-API-Key) | A secret the calling system sends with every request; a missing or wrong key is refused. |
| Branch / main | A branch is a separate line of the code; main is the main line, which is deployed. elm-optimized is a newer branch not yet merged into main. |
| Budget cap | A daily limit on model spending ($25 by default); model calls stop until midnight India time once it is reached. |
| CI | Continuous integration: automatic tests on every change, and an image build on main (GitHub Actions). |
| Confidence | The model's own estimate (0 to 1) that its reading is exactly right; below 0.90 goes to review. |
| Consumer / consumerId | The meter's account, identified by the caller's id; settings and history are kept per consumer. |
| Container / Docker | A sealed package of the service and everything it needs, so it runs the same anywhere. |
| Correlation id | The id of one result (correlationId), used to send feedback on it. |
| Cumulative reading | The meter's running total of energy since installation, the one number the service reads, kept as text with leading zeros (for example 0019712.7). |
| Decimal point handling | The decimal position comes from the consumer's settings or the meter type, not from the model's view of a tiny dot; for an unknown meter only a clearly visible point counts. |
| Duplicate photo | The identical photo of the same consumer sent again within 7 days gets the earlier result, without a new paid model call. |
| ECS / EC2 / Kubernetes | Three ways to run the service: managed containers on AWS (recommended), a virtual server, or a container cluster. |
| Endpoint | One address of the API for one job, for example POST /flowvision/v1/elm/extract-reading. |
| Export register | A total of energy sent from the premises to the grid; treated as the wrong screen when labelled OBIS 2.8.x (all versions) or EXPORT, EXP or EXPORTED (latest code, elm-optimized, only). |
| Feedback | A person's confirmation or correction of a reading (POST /feedback); a confirmation, or a correction that gives the right value, becomes the consumer's trusted previous reading. |
| FlowVision | The set of meter-reading services (electricity and water flow meters) this service belongs to, kept in one repository. |
| History / billing register (H1, B1, BILL) | Screens showing stored past values, not the current total. Every version treats them as the wrong screen when the AI recognises them or the label says BILL, HIST, PREV or LAST; the latest code (elm-optimized) also rejects the labels H, H1-H12 and B1-B12. |
| Idempotency / request id | The caller's id for one photo is also the retry key: the same id returns the stored result instead of reading the photo again. |
| JalSoochak | The central system that sends the meter photos to this service and collects the readings; its app is used to take the photos. |
| kWh, kVAh | Units of electrical energy; the meter's kWh (or kVAh) total is what the consumer is billed on. These meters show both. |
| kVArh | Reactive energy: a cumulative register too, but not the target, so it counts as the wrong screen. |
| LCD display | The small digital display on the meter; the service reads only digital meters. |
| Load balancer (ALB) | The front door that passes requests to the service; its idle timeout must be 120 s. |
| Maximum demand (MD) | The highest power drawn in a period, shown in kW or kVA; not an energy total, so a retake. |
| Median | The middle value: half the photos were faster (or cheaper) than it, half slower (or dearer). |
| Meter type | A known meter model with a fixed decimal position (and usually digit count), recognised from its printed model code. |
| Model / provider | The AI that reads the photo, and the company that runs it: OpenAI (GPT-6 Sol) or Anthropic (Claude Sonnet 5.5). |
| OBIS code | A standard numbering of meter registers; 1.8.1 to 1.8.9 are tariff totals and 2.8.x export, while 1.8.0 passes as the total. |
| OCR | Optical character recognition: reading text or digits from a picture. |
| PostgreSQL (RDS) | The production database that stores each reading request, its result and feedback. |
| Prompt | The fixed written instructions sent to the model with every photo. |
| RETAKE (UNCLEAR / NOMETER) | The photo cannot give the right reading: ask for a new photo. |
| Retake hint | The fixed message shown to the person taking the photo, telling them how to take a better one. |
| REVIEW (NEEDS_REVIEW) | The photo may give the right reading, but a person must confirm or correct it. |
| REVIEW queue | The REVIEW results waiting for a person to confirm or correct them. |
| S3 | Amazon's file storage, where photos are kept with public access blocked. |
| Screen label | The small text next to the number (for example CUM kWh, T1, MD kW) that tells which screen is showing. |
| Seven-segment digits | Digits drawn from seven bars; glare can make some look alike (0/8, 3/8/9, 5/6, 1/7, 2/7). |
| Tariff / time-of-day register | A total for one tariff zone (T1, T2, TOD...), which holds only part of the energy. |
| Token | The unit AI providers count and bill text and images in; the service prices $2 per million input and $10 per million output tokens. |
| Trusted previous reading | The latest reading of the consumer that was accepted, confirmed or corrected; new readings are compared with it. |
| uncertainDigits | The positions of digits the model was unsure of, so the reviewer knows which to check first. |
| Vision model | An AI model that can look at a picture and answer questions about it. |

# Appendix B: Every reason code

reasons[].message and retakeHint are fixed texts, returned exactly as below. A RETAKE carries the hint of its first reason; a REVIEW or ACCEPT has no hint. An ACCEPT (SUCCESS) has no reason.

| Code | Answer (status) | Message (reasons[].message) | Retake hint (retakeHint) |
| --- | --- | --- | --- |
| NO_DISPLAY | RETAKE (NOMETER) | No meter display is visible, or it is blank. | Take the photo straight at the meter's display, with the display switched on. |
| WRONG_SCREEN | RETAKE (UNCLEAR) | The display is showing another screen, not the cumulative energy reading. | Wait until the display shows the total kWh (or kVAh) reading, then take the photo. |
| UNIT_NOT_VISIBLE | RETAKE (UNCLEAR) | No kWh or kVAh label was seen, so it is not certain which energy total is shown (these meters show both). | Take the photo so the whole display is visible, including the kWh or kVAh label next to the reading. |
| UNIT_MISMATCH | RETAKE (UNCLEAR) | The display shows a different unit from the one billed for this meter. | Wait until the display shows the billed unit (kWh or kVAh), then take the photo. |
| UNREADABLE | REVIEW | The reading could not be read with certainty. | none |
| DIGITS_UNCLEAR | REVIEW | One or more digits are unclear, or a digit may be missing. | none |
| LOW_CONFIDENCE | REVIEW | The model was not confident enough in the reading. | none |
| BAD_OUTPUT | REVIEW | The model's answer was internally inconsistent. | none |
| DIGIT_COUNT_MISMATCH | REVIEW | The number of digits does not match this meter type; a digit may have been dropped. | none |
| DECIMAL_UNCERTAIN | REVIEW | Meter type unknown and no decimal point was seen, so the decimal position is uncertain. Set this consumer's meter type to accept readings automatically. | none |
| NO_CROSS_CHECK | REVIEW | The model was not confident enough to accept this reading on its own, and this meter has no known digit count or previous reading to cross-check it. Set the consumer's meter type, or confirm this reading through feedback. | none |
| BELOW_PREVIOUS | REVIEW | The reading is lower than the previous accepted reading. | none |
| JUMP_TOO_LARGE | REVIEW | The increase since the previous reading is implausibly large. | none |
| READS_DISAGREE | REVIEW | Independent reads of the same photo gave different results. | none |
| HISTORY_UNAVAILABLE | REVIEW | The consumer's previous readings could not be checked (the database did not answer), so the reading was not compared with them. | none |
| CORRECTED_BEFORE | REVIEW | A person marked the reading of this same photo wrong or corrected it, so it is not read or accepted again; the earlier model reading is shown. | none |

When there is no number to show, meterReading holds one of these fixed sentences:

| Status | meterReading when there is no number |
| --- | --- |
| NOMETER | No meter display detected in the image (always, since there is no display) |
| UNCLEAR | Not the cumulative energy reading; please retake the photo |
| NEEDS_REVIEW | Reading needs manual review |

# Appendix C: Where to find more

| You want to... | Read (in services/elm-ocr) |
| --- | --- |
| Understand the service, run it locally, or change the code | README.md |
| Call the API from another system: every request field, every response, errors and retries | docs/INTEGRATION.md |
| Deploy and operate it: AWS resources, settings, alarms, limits, runbook, go-live checklist | docs/DEPLOY_AWS.md |
| Deploy on Kubernetes, or understand the repository layout | docs/adr/ADR-001-multi-service-layout.md (at the repository root) |
| See every setting | src/conf/config.yaml (with the environment variable for each setting that has one), .env.example, and gunicorn.conf.py (server settings); the few environment-only settings are read in src/conf/config.py, src/conf/logging.py and src/service/api/database.py |
| See the exact rules | src/service/vision/checks.py (checks and reason codes), meter_types.py (meter types), prompt.py (the prompt) |
| Try the API interactively | /docs on a service running in development mode (off in production) |